PowerWave Analyst

Document 3 of 3 — Detailed Implementation Plan

Revision 2 — Updated milestones for display mode, bay grouping, complementary pairs

What Changed in Revision 2 Updated: Milestone 1A — data models now include display_mode, bay_name, is_derived, is_complementary fields. Updated: Milestone 1B — COMTRADE parser must handle safe field-count parsing, bay extraction, display mode determination, non-zero offset application, multi-rate time array, and complementary pair detection. Updated: Milestone 1C — CSV/Excel parser must detect slow-rate records and set TREND mode. Updated: Milestone X1 (signal detector) — must handle R/Y/B, Chinese naming, generator channel types, COMMFAIL exception. Added: Acceptance tests using real file names.

# 1. Updated Milestone 1A — Data Models

Build first. All other code depends on these being correct. Rev 2 adds new fields confirmed from real file analysis.

## 1.1 src/models/channel.py

- Create AnalogueChannel dataclass with ALL fields from Document 2 Section 1.2 including bay_name and is_derived

- Create DigitalChannel dataclass with ALL fields from Document 2 Section 1.3 including bay_name, is_complementary, complementary_id

- Add signal role constants matching taxonomy in Document 2 Section 1.4 exactly

- Validation in __post_init__: offset may be any float including non-zero — do not enforce zero

- Validation: primary and secondary accept None (NARI case) as well as float

## 1.2 src/models/disturbance_record.py

- Create DisturbanceRecord dataclass with ALL fields from Document 2 Section 1.1 including display_mode and bay_names

- display_mode must default to WAVEFORM — parser sets it based on sample_rate

- bay_names is a list[str] — populated by parser from channel name extraction

- time_array must be float64 — explicitly enforce this dtype

- _rms_cache and _phasor_cache initialised as empty dicts in __post_init__

Acceptance criteria:

- DisturbanceRecord with display_mode='TREND' and display_mode='WAVEFORM' both instantiate correctly

- AnalogueChannel with offset=-1875.0 (field current case) instantiates without validation error

- AnalogueChannel with primary=None (NARI case) instantiates without validation error

- DigitalChannel with is_complementary=True and complementary_id=2 instantiates correctly

# 2. Updated Milestone 1B — COMTRADE Parser

This milestone is the most complex in the project. Must handle all vendor quirks confirmed from real files.

## 2.1 CFG Parser — Complete Requirements

- Safe rev_year parsing: if absent or not in {1991,1999,2013} → default to 1999

- Safe analogue line parsing: handle both 10-field (NARI) and 13-field (BEN32/ABB) formats

- Safe digital line parsing: handle both 3-field (NARI) and 5-field (BEN32/ABB) formats

- Apply physical = (raw × multiplier) + offset to every channel — offset ALWAYS applied

- Multi-rate time array: build section-by-section for non-uniform spacing (see SKILL_comtrade_parser.md)

- Display mode: set WAVEFORM if primary rate ≥ 200Hz, TREND if < 200Hz

- Bay extraction from analogue channel names — see bay extraction rules in CLAUDE.md

- Bay extraction from digital channel names using known bays from analogue parsing

- is_derived flag: set True for channels with unit MW or MVAr that contain pre-calculated power

- Complementary pair detection: find DIG_CB channels with matching base name and OPEN/CLOSED suffix

- Handle both LF and CRLF line endings

- Handle UTF-8, Latin-1, and ASCII encoding in CFG text

- Tolerate missing optional fields with sensible defaults — never crash on missing field

## 2.2 DAT Parser

- Implement ASCII, BINARY (int16), BINARY32 (int32), FLOAT32 DAT readers

- Digital channel bit extraction from uint16 words

- Timestamp monotonicity validation — warn but do not crash

Acceptance criteria — test against real files:

- BEN32 KULW_275kV.cfg: 28 analogue, 104 digital, 5000Hz, display_mode=WAVEFORM, 4 bay groups detected

- BEN32 JMHE_U2.cfg: 15 analogue, 7 digital, 50Hz, display_mode=TREND, field current offset=-1875 applied correctly

- NARI relay.cfg: 9 analogue, 32 digital, multi-rate time array non-uniform, primary=None on all channels

- Any 275kV BEN32 fast record: pre-fault RMS of VR channel ≈ 158.8kV (275/√3)

- Any BEN32 slow record: MW channel is_derived=True, physical value plausible for generator output

# 3. Updated Milestone X1 — Signal Role Detector

Implement immediately after all parsers complete (end of Phase 1). Required before any analysis engine.

- Implement src/parsers/signal_role_detector.py with 8-priority detection rules per SKILL_channel_mapping.md

- Priority 1: unit field — most reliable, handles kV/kA/Hz/MW/MVAr/RPM/%

- Priority 2: non-zero offset signature — identifies DC_FIELD_I, DC_FIELD_V, MECH_SPEED, MECH_VALVE

- Priority 3: power channel names — 'POWER UNIT', 'R.POWER', 'ACTIVE POWER'

- Priority 4: frequency channel names — 'FREQ', 'FREQUENCY', 'DF/DT'

- Priority 5: sequence RMS names — 'O SEQ RMS', 'ZERO SEQ', 'NEG SEQ'

- Priority 6: direct signal code lookup table — R/Y/B, A/B/C, Chinese Ia/Ub/3I0 — all conventions

- Priority 7: pattern matching on full name

- Priority 8: magnitude heuristic fallback

- Digital detection: ALARM EXCEPTION checked FIRST — COMMFAIL/FAIL/ALARM → DIG_GENERIC always

- Digital: CB → AR → INTERTRIP → TRIGGER → TRIP → PICKUP priority order

- Phase extraction must support R/Y/B → A/B/C mapping AND a/b/c → A/B/C mapping

Acceptance criteria:

- BEN32 'SJTC2 VR' → V_PHASE, phase=A, confidence=HIGH

- BEN32 'KPAR IY' → I_PHASE, phase=B, confidence=HIGH

- BEN32 'JMHE FIELD CURRENT' (offset=-1875) → DC_FIELD_I, confidence=HIGH

- BEN32 'POWER UNIT UNIT NO.2' → P_MW, is_derived=True, confidence=HIGH

- NARI 'Ia' → I_PHASE, phase=A, confidence=HIGH

- NARI '3I0' → I_EARTH, phase=N, confidence=HIGH

- NARI 'Ux' → V_RESIDUAL, phase=N, confidence=HIGH

- BEN32 'SJTC2 87L/1_COMMFAIL' → DIG_GENERIC (COMMFAIL exception — NOT DIG_TRIP)

- BEN32 'SJTC2 87L/1' → DIG_TRIP, confidence=HIGH

- BEN32 'SJTC2 CB_R OPEN' → DIG_CB, confidence=HIGH

- BEN32 'SJTC2 79AR_ATTEMPTED' → DIG_AR, confidence=HIGH

- BEN32 'SJTC2 50BF_STG1' → DIG_INTERTRIP, confidence=HIGH

- NARI 'Op_Z1' → DIG_TRIP, confidence=HIGH

- NARI 'BI_MCB_VT' → DIG_GENERIC (alarm exception), confidence=HIGH

# 4. Updated Milestone X2 — Channel Mapping Dialog

Implement at start of Phase 2. The SIGRA-equivalent manual identification fallback. All analysis engines depend on signal roles being correct.

- Create src/ui/channel_mapping_dialog.py as QDialog

- Scrollable channel table: one row per channel with original name, data preview (5 values), signal role dropdown, phase dropdown, unit field, confidence badge (colour-coded HIGH=green, MEDIUM=amber, LOW=red)

- Signal role dropdown populated from complete taxonomy in Document 2 Section 1.4

- Phase dropdown: A/B/C/N/AB/BC/CA/Pos-seq — shown only for roles with phase variants

- Bulk assignment: Shift+click and Ctrl+click multi-row selection then assign role to all selected

- Auto-detection suggestions shown in italic — HIGH confidence pre-ticked, MEDIUM/LOW not pre-ticked

- Save Profile button: saves mapping as JSON keyed by station_name+device_id

- Auto-apply saved profile on next matching file load — show notification banner, skip dialog

- Profile manager: Edit → Channel Mapping Profiles → list/rename/delete

- Non-blocking validation warnings: duplicate phase, incomplete 3-phase set, missing V+I pair

- Confirm and Cancel buttons

Show dialog automatically when:

- Any CSV or Excel file loaded — always, no naming standard exists

- COMTRADE where more than 20% of analogue channels are LOW confidence

- PMU CSV where column auto-detection failed or returned None

- User right-clicks channel in panel and selects Reassign Signal Type

- User selects Edit → Channel Mapping from menu

Do NOT show dialog for:

- Well-formed COMTRADE where all channels detected at HIGH confidence

- Any file matching a previously saved mapping profile

Acceptance criteria:

- Load BEN32 KULW_275kV.cfg → dialog does NOT appear (all channels HIGH confidence)

- Load a generic CSV with non-standard headers → dialog appears automatically

- Save mapping profile for NARI relay → reload same file → dialog skipped, banner shown

- Assign DIG_GENERIC to a channel previously detected as DIG_TRIP → role updated immediately in channel panel